| CS 207D- Computer Architecture Second Semester 1434/1435H Sheet #2 | Princess Noura Bint Abdulrahman University Faculty of Computer and Information Sciences Department of the Computer Sciences ( |
| --- | --- |

Q1: What are the fundamental and basic design principles in MIPS-based computer architecture?

Q2 : Write the following sequence of code into MIPS assembly language

- x = x + y + z - q;

Assume that x, y, z, q are stored in registers $s1-$s4

2-a=b-c,  d=3*a  assume that the variables a ,b ,c and d are associated with registers $s0,$s1,$s2 and $s3

3-G=2*F   , A[17]=A[16]+G

Assume that  A  is an array of 32-bit numbers whose base address is stored in $s0

Q3 : In the following MIPS  code :

- How many times is instruction memory accessed?

- How many times is data memory accessed? (Count only accesses to memory, not registers.)

lw $v1, 0($a0)

addi $v0, $v0, 1

sw $v1, 0($a1)

addi $a0, $a0, 1

Q4 : Use the register and memory values in the table below for the next questions:

Assume a  32-bit machine. Assume each of the following questions starts from the table values; that is, DO NOT use value changes from one question as propagating into future parts of the questions

- What values will be in R1 and R3 after this instruction is executed:

load R3, 12(R1)

- What values will be in the registers after this instruction is executed:

addi R2, R3, #16

- Give the values of R1, R2, and R3 after this instruction:

add R3, R2, R1